| MSPL |  | Week 2 |  | 何柏昇 |
| --- | --- | --- | --- | --- |
| Denoise |  |  |  |  |  |
| Mean Filter |  |  |  |  |  |
| 平滑降噪，原理主要是計算該點周圍的平均值，來達到降低雜訊，但也會使圖片模糊化。 |  |  |  |  |
| Median Filter |  |  |  |  |
| 取其與周圍的鄰近，找出中位數並以其取代該點，其優點在於不會使圖片過度模糊化。 |  |  |  |  |
| Gaussian Filter |  |  |  |  |
| 減少圖像雜訊以及降低細節層次，其視覺效果就像是經過一個半透明屏幕在觀察圖像。少了很多顆粒感，但也變模糊了。 |  |  |  |  |
| Gx,y=12πσ2e-x2+y22σ2 |  |  |  |  |

// Denoise: Mean, Median, Gaussian #include <stdio.h> #include <ctime> #include <cmath> #include "opencv2/opencv.hpp" using namespace std; using namespace cv; //Mean Filter void meanFilter(Mat &src, Mat &dst, int kernel) { if (!src.data) return; //At pixel of image for (int y = 0; y < src.rows; y++) { for (int x = 0; x < src.cols; x++) { int sum; if ((y-kernel >= 0)&&(x-kernel >= 0)&&(y+kernel < src.rows)&&(x+kernel < src.cols)) { // Sum pixels in the kernel size for (int k1 = 0; k1 < (2*kernel+1); k1++) for (int k2 = 0; k2 < (2*kernel+1); k2++) sum += src.at<uchar>(y-kernel+k1, x-kernel+k2); // Calculate the mean value sum /= ((2*kernel+1)*(2*kernel+1)); dst.at<uchar>(y,x)=sum; } } } } //Median Filter void medianFilter(Mat &src, Mat &dst, int kernel) { if (!src.data) return; //At pixel of image for (int y = 0; y < src.rows; y++) { for (int x = 0; x < src.cols; x++) { int tmp[(2*kernel+1)*(2*kernel+1)]; if ((y-kernel >= 0)&&(x-kernel >= 0)&&(y+kernel < src.rows)&&(x+kernel < src.cols)) { // List pixels in the kernel size for (int k1 = 0; k1 < (2*kernel+1); k1++) for (int k2 = 0; k2 < (2*kernel+1); k2++) tmp[k1 * (2*kernel+1) + k2] = src.at<uchar>(y-kernel+k1, x-kernel+k2); // Finde the median value sort(tmp,tmp+((2*kernel+1)*(2*kernel+1))); dst.at<uchar>(y, x)=tmp[((2*kernel+1)*(2*kernel+1)-1)/2]; } } } } // Gaussian Filter void gaussianFilter(Mat &src, Mat &dst, int kernel, double sigma= 0.84089642) { // Initialise values double GKernel[2*kernel+1][2*kernel+1]; double r, s=2.0 * sigma * sigma; double sum=0.0; // Generate the gaussian kernel for (int y = 0; y < (2*kernel+1); y++) { for (int x = 0; x < (2*kernel+1); x++) { r = ((x-kernel)*(x-kernel)+(y-kernel)*(y-kernel)); GKernel[y][x] = (exp(-r/s)) / (M_PI * s); sum += GKernel[y][x]; } } // Normalize the kernel for (int y = 0; y < (2*kernel+1); y++) for (int x = 0; x < (2*kernel+1); x++) {GKernel[y][x] /= sum; //cout << GKernel[y][x]<<endl; } // Convolution image with GKernel for (int y = 0; y < src.rows; y++) { for (int x = 0; x < src.cols; x++) { if ((y-kernel >= 0)&&(x-kernel >= 0)&&(y+kernel < src.rows)&&(x+kernel < src.cols)) { // Convolution double sum = 0; for (int k1 = 0; k1 < (2*kernel+1); k1++) for (int k2 = 0; k2 < (2*kernel+1); k2++) sum += (double)src.at<uchar>(y-kernel+k1, x-kernel+k2)*GKernel[k1][k2]; dst.at<uchar>(y,x)=sum; } } } } // Salt the image void salt(Mat &image, int num) { if (!image.data) return; int x, y; srand(time(NULL)); for (int i = 0; i < num; i++) { x = rand() % image.rows; y = rand() % image.cols; image.at<uchar>(y,x) = 255; } } int main(int argc, char** argv) { // Read the image Mat image = imread("../images/lena.png",0); // Add the salt noise to image Mat saltImage = image.clone(); salt(saltImage, 5000); // Setting the kernel size if ( argc != 2 ¦¦ (atoi(argv[1])%2) == 0 ¦¦ atoi(argv[1]) > image.rows ¦¦ atoi(argv[1]) > image.cols) { printf("usage: ./denoise <kernel size(3 ,5 ,7, ..., 2N+1)>\n"); return -1; } int kernel; kernel = (atoi(argv[1])-1)/2; // Mean Filter Mat meanImage = image.clone(); meanFilter(saltImage, meanImage, kernel); // Median Filter Mat medianImage = image.clone(); medianFilter(saltImage, medianImage, kernel); // Gaussian Filter Mat gaussianImage = image.clone(); gaussianFilter(saltImage, gaussianImage, kernel); // Show images imshow("Original",image); imshow("Salt",saltImage); imshow("Mean",meanImage); imshow("Median",medianImage); imshow("Gaussian",gaussianImage); waitKey(); // Save images imwrite("../images/salt.png", saltImage); imwrite("../images/mean.png", meanImage); imwrite("../images/median.png", medianImage); imwrite("../images/gaussian.png", gaussianImage); return 0; }

| Original |  |
| --- | --- |
| Salt | Mean |
| Median | Gaussian |

| Edge Detection |  |  |  |
| --- | --- | --- | --- |
| Sobel |  |  |  |  |
| 分別計算： - 偏x方向的Gx - 偏y方向的 Gy - 求絕對值，壓縮到[0, 255]區間 Gx,y=Gx+Gy |  |  |  |  |
| Laplacian |  |  |  |  |
| 一階差分： f'x=fx-fx-1 二階差分： f'x=fx+1-fx-(f(x)-fx-1 =fx-1-2fx+f(x+1) 二維: f'x,y=-4fx,y+fx-1,y+fx+1,y+fx,y-1+fx,y+1 |  |  |
| Canny |  |  |  |  |
| 1.高斯濾波器平滑圖像。 2.一階差分偏導計算梯度值和方向Sobel。 3.對梯度值不是極大值的地方進行抑制。 4.用雙閾值連接圖上的聯通點。 |  |  |  |  |

// Edge Detect: Sobel, Laplacian #include <stdio.h> #include <cmath> #include "opencv2/opencv.hpp" using namespace std; using namespace cv; void sobelKernel(Mat &src, Mat &dst, int kernel, int sita) { // Initialise values int size= 2 * kernel + 1; double Gx[size][size], Gy[size][size], G[size][size]; // Generate sobel kernel for (int y = 0; y < size; y++) { for (int x = 0; x < size; x++) { if (y != kernel ¦¦ x != kernel) { // Gx f(x,y) dot (1,0) Gx[y][x] = (double)(x-kernel) / (double)((x-kernel)*(x-kernel) + (y-kernel)*(y-kernel)); // Gy f(x,y) dot (0,1) Gy[y][x] = (double)(y-kernel) / (double)((x-kernel)*(x-kernel) + (y-kernel)*(y-kernel)); } else Gx[y][x]=Gy[y][x]=0; // G = (Gx*cosa + Gy*sina)/2 G[y][x] = (Gx[y][x] * cos(sita) + Gy[y][x] * sin(sita)); } } // Convolution image with soble kernel for (int y = 0; y < src.rows; y++) { for (int x = 0; x < src.cols; x++) { if ((y-kernel >= 0)&&(x-kernel >= 0)&&(y+kernel < src.rows)&&(x+kernel < src.cols)) { // Convolution double sum = 0; for (int k1 = 0; k1 < size; k1++) for (int k2 = 0; k2 < size; k2++) sum += src.at<uchar>(y-kernel+k1, x-kernel+k2) * G[k1][k2]; dst.at<uchar>(y,x)=abs(sum); } else dst.at<uchar>(y,x)=0; } } } // Combine two images to one void addImage(Mat &src1, Mat &src2,Mat &dst) { for (int y = 0; y < dst.rows; y++) for (int x = 0; x < dst.cols; x++) dst.at<uchar>(y,x)=sqrt(pow(src1.at<uchar>(y,x),2) + pow(src2.at<uchar>(y,x),2)); } // Laplacian Filter void laplacianFilter(Mat &src, Mat &dst, int kernel,double sigma = 84089642) { // Initialise values int size = 2 * kernel + 1; double LKernel[size][size]; // Generate sobel kernel for (int y = 0; y < size; y++) { for (int x = 0; x < size; x++) { if (y != kernel ¦¦ x != kernel) LKernel[y][x] = -1; else LKernel[y][x] = size * size - 1; } } // Convolution image with GKernel for (int y = 0; y < src.rows; y++) { for (int x = 0; x < src.cols; x++) { if ((y-kernel >= 0)&&(x-kernel >= 0)&&(y+kernel < src.rows)&&(x+kernel < src.cols)) { // Convolution double sum = 0; for (int k1 = 0; k1 < size; k1++) for (int k2 = 0; k2 < size; k2++) sum += (double)src.at<uchar>(y-kernel+k1, x-kernel+k2)*LKernel[k1][k2]; dst.at<uchar>(y,x)=abs(sum); } else dst.at<uchar>(y,x) = 0; } } } int main(int argc, char** argv) { // Read the image Mat image = imread("../images/lena.png",0); // Setting the kernel size if ( argc != 2 ¦¦ (atoi(argv[1])%2) == 0 ¦¦ atoi(argv[1]) > image.rows ¦¦ atoi(argv[1]) > image.cols) { printf("usage: ./edgeDetect <kernel size(3 ,5 ,7, ..., 2N+1)>\n"); return -1; } int kernel; kernel = (atoi(argv[1])-1)/2; // Sobel Mat sobelImageX = image.clone(), sobelImageY = image.clone(), sobelImage = image.clone(); sobelKernel(image, sobelImageX, kernel, (0*M_PI/180)); sobelKernel(image, sobelImageY, kernel, (90*M_PI/180)); addImage(sobelImageX, sobelImageY, sobelImage); // Laplacian of Gaussian Mat LaplacianImage = image.clone(); laplacianFilter(image,LaplacianImage,kernel); // Show images imshow("Original",image); imshow("Sobel",sobelImage); imshow("Laplacian", LaplacianImage); waitKey(); // Save images imwrite("../images/sobel.png", sobelImage); imwrite("../images/sobelX.png", sobelImageX); imwrite("../images/sobelY.png", sobelImageY); imwrite("../images/laplacian.png", LaplacianImage); }

Original

Sobel

Laplacian

/* Edge Detect: Canny * Step 1: Apply a Gaussian blur * Step 2: Find edge gradient strength and direction * Step 3: Trace along the edges * Step 4: Suppress non-maximum edges */ #include <stdio.h> #include <unistd.h> #include <cmath> #include "opencv2/opencv.hpp" using namespace std; using namespace cv; void gaussianFilter(Mat &src, Mat &dst, int kernel, double sigma= 1.4) { // Initialise values int size = 2 * kernel + 1; double GKernel[size][size]; double r = 0, s=2.0 * sigma * sigma, sum = 0; // Gaussian Kernel for (int i = 0; i < size; i++) { for (int j = 0; j < size; j++) { int y = i -kernel, x = j - kernel; r = pow(x,2) + pow(y,2); double normal = 1 / (s * M_PI); GKernel[y][x] = exp(-(r) / s) * normal; sum += GKernel[y][x]; } } // Normalize the kernel for (int y = 0; y < size; y++) for (int x = 0; x < size; x++) GKernel[y][x] /= sum; // Convolution image with GKernel for (int y = 0; y < src.rows; y++) { for (int x = 0; x < src.cols; x++) { if ((y-kernel >= 0)&&(x-kernel >= 0)&&(y+kernel < src.rows)&&(x+kernel < src.cols)) { // Convolution sum = 0; for (int k1 = 0; k1 < (size); k1++) for (int k2 = 0; k2 < (size); k2++) sum += (double)src.at<uchar>(y-kernel+k1, x-kernel+k2)*GKernel[k1][k2]; dst.at<uchar>(y,x)=sum; } } } } void sobelFilter(Mat &src, Mat &dst, Mat &dst2,int kernel) { // Initialise values int size= 2 * kernel + 1; double Gx[size][size], Gy[size][size]; double angle[src.rows][src.cols]; double G[src.rows][src.cols]; double Gmax=0.0; double sum = 0.0, sumX = 0.0, sumY = 0.0; for (int i = 0; i < src.rows; i++) { for (int j = 0; j < src.cols; j++) { angle[j][i] = 0.0; G[j][i] = 0.0; } } // Sobel kernel for (int y = 0; y < size; y++) { for (int x = 0; x < size; x++) { if (y != kernel ¦¦ x != kernel) { // Gx f(x,y) dot (1,0) Gx[y][x] = (double)(x-kernel) / (double)((x-kernel)*(x-kernel) + (y-kernel)*(y-kernel)); // Gy f(x,y) dot (0,1) Gy[y][x] = (double)(y-kernel) / (double)((x-kernel)*(x-kernel) + (y-kernel)*(y-kernel)); } else Gx[y][x]=Gy[y][x]=0; } } // Convolution image with Gx and Gy for (int y = 0; y < src.rows; y++) { for (int x = 0; x < src.cols; x++) { if ((y-kernel > 0)&&(x-kernel > 0)&&(y+kernel < src.rows)&&(x+kernel < src.cols)) { // Convolution sumX = 0, sumY=0; for (int k1 = 0; k1 < (size); k1++) for (int k2 = 0; k2 < (size); k2++) { sumX += src.at<uchar>(y-kernel+k1, x-kernel+k2)*Gx[k1][k2]; sumY += src.at<uchar>(y-kernel+k1, x-kernel+k2)*Gy[k1][k2]; } G[y][x] = sqrt(pow(sumX,2) + pow(sumY,2)); if (G[y][x] > Gmax) Gmax = G[y][x]; angle[y][x] = (atan2(sumX,sumY) / M_PI) * 180.0; if (angle[y][x] < 0) angle[y][x] += 180; } } } // Normalize for (int y = 0; y < src.rows; y++) for (int x = 0; x < src.cols; x++) dst.at<uchar>(y,x) = (uchar)(G[y][x] * (255/Gmax)); // Non Max Suppression for (int y = 0; y < src.rows; y++) { for (int x = 0; x < src.cols; x++) { if ((y-kernel >= 0)&&(x-kernel >= 0)&&(y+kernel < src.rows)&&(x+kernel < src.cols)) { int q = 255, r = 255; if ((0 <= angle[y][x] < 22.5) ¦¦ (157.5 <= angle[y][x] <= 180)) { q = dst.at<uchar>(y,x+1); r = dst.at<uchar>(y,x-1); } else if ((22.5 <= angle[y][x]) && (angle[y][x] < 67.5)) { q = dst.at<uchar>(y+1,x-1); r = dst.at<uchar>(y-1,x+1); } else if ((67.5 <= angle[y][x]) && (angle[y][x] < 112.5)) { q = dst.at<uchar>(y+1,x); r = dst.at<uchar>(y-1,x); } else if ((112.5 <= angle[y][x]) && (angle[y][x] < 157.5)) { q = dst.at<uchar>(y-1,x-1); r = dst.at<uchar>(y+1,x+1); } if ((dst.at<uchar>(y,x) >= q) && (dst.at<uchar>(y,x) >= r)) dst2.at<uchar>(y,x) = dst.at<uchar>(y,x); else dst2.at<uchar>(y,x) = 0; } else dst2.at<uchar>(y,x) = 0; } } } // double thershold void db_thershold(Mat &src, Mat &dst, int kernel,int low, int high) { int G[src.rows][src.cols]; int size = 2 * kernel + 1; for (int y = 0; y < src.rows; y++) { for (int x = 0; x < src.cols; x++) { if ((y-kernel >= 0)&&(x-kernel >= 0)&&(y+kernel < src.rows)&&(x+kernel < src.cols)) { if (src.at<uchar>(y,x)<low) G[y][x] = 0; else if ((src.at<uchar>(y,x)>low) && (src.at<uchar>(y,x)<high)) { bool edge = false; for (int k1 = 0; k1 < (size); k1++) { for (int k2 = 0; k2 < (size); k2++) { if (src.at<uchar>(y-kernel+k1, x-kernel+k2)>high) { edge = true; break; } } } if (!edge) G[y][x] = 0; } dst.at<uchar>(y,x) = G[y][x]; } } } } int main(int argc, char** argv) { // Read the image Mat image = imread("../images/lena.png",0); Mat gaussianImage = image.clone(); Mat sobelImage = image.clone(); Mat suppressionImage = image.clone(); Mat dbImage = suppressionImage.clone(); // Gaussian int kernel; printf("Enter kernel size of gaussian filter(3 ,5 ,7, ..., 2N+1): "); scanf("%d",&kernel); if ((kernel%2) == 0 ¦¦ kernel > image.rows ¦¦ kernel > image.cols ¦¦ kernel == 1) { printf("Error value!!\n"); return -1; } kernel = (kernel-1)/2; gaussianFilter(image, gaussianImage, kernel); // Sobel printf("Enter kernel size of sobel filter(3 ,5 ,7, ..., 2N+1): "); scanf("%d",&kernel); if ((kernel%2) == 0 ¦¦ kernel > image.rows ¦¦ kernel > image.cols ¦¦ kernel == 1) { printf("Error value!!\n"); return -1; } kernel = (kernel-1)/2; sobelFilter(gaussianImage, sobelImage, suppressionImage, kernel); // Double thershold printf("Enter kernel size of double thershold(3 ,5 ,7, ..., 2N+1): "); scanf("%d",&kernel); if ((kernel%2) == 0 ¦¦ kernel > image.rows ¦¦ kernel > image.cols ¦¦ kernel == 1) { printf("Error value!!\n"); return -1; } kernel = (kernel-1)/2; int low=0, high=0; printf("Enter low and high value for double thershold: "); scanf("%d %d",&low, &high); if (low > high ¦¦ high > 254 ¦¦ low < 0) { printf("Error value!!%d %d\n",low ,high); return -1; } db_thershold(suppressionImage, dbImage, kernel, low, high); // Show images /*imshow("Original",image); imshow("Gaussian", gaussianImage); imshow("Sobel", sobelImage); imshow("Suppression", suppressionImage); imshow("Double thershold", dbImage); waitKey();*/ // Save images imwrite("../images/1-gaussian.png", gaussianImage); imwrite("../images/2-sobel.png", sobelImage); imwrite("../images/3-suppression.png", suppressionImage); imwrite("../images/4-dbThershold.png", dbImage); return 0; }

| 1-Gaussian | 2-Sobel |
| --- | --- |
| 3-Supperssion | 4-Double thershold |

Reference. - https://blog.csdn.net/qinghuaci666/article/details/81737624?spm=1001.2101.3001.6661.1&utm_medium=distribute.pc_relevant_t0.none-task-blog-2%7Edefault%7ECTRLIST%7Edefault-1-81737624-blog-7297736.pc_relevant_default&depth_1-utm_source=distribute.pc_relevant_t0.none-task-blog-2%7Edefault%7ECTRLIST%7Edefault-1-81737624-blog-7297736.pc_relevant_default&utm_relevant_index=1 - http://www.librow.com/articles/article-1 - https://www.geeksforgeeks.org/gaussian-filter-generation-c/ - https://blog.csdn.net/rocky_shared_image/article/details/7238796 - https://medium.com/@bob800530/python-gaussian-filter-%E6%A6%82%E5%BF%B5%E8%88%87%E5%AF%A6%E4%BD%9C-676aac52ea17 - https://www.twblogs.net/a/5b7ac2042b7177392c96c153 - https://stackoverflow.com/questions/9567882/sobel-filter-kernel-of-large-size - https://docs.opencv.org/3.4/d2/d2c/tutorial_sobel_derivatives.html - https://medium.com/%E9%9B%BB%E8%85%A6%E8%A6%96%E8%A6%BA/%E9%82%8A%E7%B7%A3%E5%81%B5%E6%B8%AC-%E6%8B%89%E6%99%AE%E6%8B%89%E6%96%AF%E7%AE%97%E5%AD%90-laplacian-operator-ea877f1945a0 - https://docs.opencv.org/3.4/d5/db5/tutorial_laplace_operator.html - https://medium.com/@bob800530/opencv-%E5%AF%A6%E4%BD%9C%E9%82%8A%E7%B7%A3%E5%81%B5%E6%B8%AC-canny%E6%BC%94%E7%AE%97%E6%B3%95-d6e0b92c0aa3. Github

MSPL/ week2